# Stage 2 Energy-Aware Policy Check

Physical node telemetry with simulated battery level
Generated 2026-10-24 22:45

# 1. Scope

This report checks mode-dependent sample intervals and confidence-indicator PWM caps using node telemetry. The firmware battery percentage is randomly simulated; no battery voltage, current, or energy consumption was measured.

| Node | Rows | Duration (s) | Median interval (ms) | Battery start/end (%) | LDR10 range |
| --- | --- | --- | --- | --- | --- |
| 1 | 50 | 548.0 | 14996 | 79/0 | 339–726 |
| 2 | 51 | 821.4 | 14998 | 79/0 | 301–330 |

| Node | Mode | Rows | Battery range (%) | PWM max/cap | Cap pass (%) | Interval expected/observed (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NORMAL | 10 | 61–79 | 218/255 | 100.0 | 2000/2000.257 |
| 1 | ECO | 7 | 26–57 | 128/128 | 100.0 | 5000/5000.841 |
| 1 | CRITICAL | 33 | 0–4 | 0/0 | 100.0 | 15000/15000.032 |
| 2 | NORMAL | 7 | 65–79 | 222/255 | 100.0 | 2000/2000.7939999999999 |
| 2 | ECO | 11 | 21–59 | 128/128 | 100.0 | 5000/5000.138 |
| 2 | CRITICAL | 33 | 0–3 | 0/0 | 100.0 | 15000/14999.91 |

Received MQTT samples per node and simulated energy mode.

# 2. Interpretation

Passing these checks demonstrates the firmware's simulated energy policy only. It does not demonstrate battery life, measured energy savings, or performance under a physical low-voltage condition.