# Step S1.2 Report: Model Training and Comparison

TinyFL / Stage 1, regression of LED PWM from PIR, LDR, temperature and humidity
Generated 2026-10-10 19:00

# 1. Purpose

Compare candidate regressors on the S1.1 dataset with evidence rather than assumption, and fix the model that will be exported to C for the ESP32-S3.

# 2. Method

The dataset (S1.1) was split 80/20 at random with seed 42 (8000 training, 2000 test samples). Each candidate was scored by 5-fold cross-validation on the training part, then refitted on the whole training part and scored once on the held-out test part. Feature order is [PIR, LDR, temperature, humidity], matching the firmware. The Random Forest uses 10 trees, maximum depth 5 (small enough for a microcontroller). The decision tree uses depth 5. KNN (k = 5) is reported both on raw features, as in the published paper, and on standardised features, because unscaled distances are dominated by the 0-1023 LDR range.

# 3. Results

## 3.1 Cross-validation

| Model | MSE (mean +/- SD) | RMSE | R2 (mean +/- SD) |
| --- | --- | --- | --- |
| Random Forest (10 trees, depth 5) | 567.57 +/- 131.63 | 23.82 | 0.9403 +/- 0.0138 |
| Decision Tree (depth 5) | 587.25 +/- 131.61 | 24.23 | 0.9383 +/- 0.0135 |
| Linear Regression | 3472.11 +/- 104.53 | 58.92 | 0.6345 +/- 0.0076 |
| KNN k=5 (unscaled, paper protocol) | 7775.54 +/- 135.83 | 88.18 | 0.1811 +/- 0.0176 |
| KNN k=5 (standardised) | 675.04 +/- 137.13 | 25.98 | 0.9290 +/- 0.0143 |

Table 1. 5-fold cross-validation on the training set.

## 3.2 Held-out test set

| Model | RMSE | MAE | R2 | Max abs error |
| --- | --- | --- | --- | --- |
| Random Forest (10 trees, depth 5) | 24.17 | 7.13 | 0.9380 | 250.6 |
| Decision Tree (depth 5) | 24.12 | 7.18 | 0.9383 | 250.8 |
| Linear Regression | 58.82 | 51.95 | 0.6330 | 201.7 |
| KNN k=5 (unscaled, paper protocol) | 85.02 | 57.63 | 0.2332 | 255.0 |
| KNN k=5 (standardised) | 26.44 | 7.65 | 0.9258 | 254.4 |

Table 2. Test-set performance of all models.

| Random Forest metric | Value |
| --- | --- |
| MSE | 584.37 |
| RMSE | 24.17 |
| MAE | 7.13 (2.8% of full scale) |
| R2 | 0.9380 |
| Explained variance | 93.8% |
| Max abs error | 250.6 |
| Error SD | 24.15 |
| Error mean | -1.01 |
| Samples with ¦error¦ > 40 | 1.25% |

Table 3. Detailed test-set metrics of the selected Random Forest.

Figure 1. Left: actual vs predicted PWM on the test set. Right: residual distribution.

## 3.3 Feature importance and learning curve

| Rank | Feature | Importance | % |
| --- | --- | --- | --- |
| 1 | ldr | 0.617 | 61.7 |
| 2 | pir | 0.380 | 38.0 |
| 3 | temp | 0.002 | 0.2 |
| 4 | hum | 0.001 | 0.1 |

Table 4. Random Forest feature importance (impurity based).

Figure 2. Feature importance.

Figure 3. Learning curve; validation R2 is within 0.005 of its final value from about 320 samples.

# 4. Comparison with the published paper

| Random Forest metric | Published (original dataset) | This rebuild (S1.1 dataset) |
| --- | --- | --- |
| R2 | 0.9789 | 0.9380 |
| RMSE | 8.98 | 24.17 |
| MAE | 4.88 | 7.13 |

Table 5. The published figures came from a different generated dataset, so differences are expected.

Do not mix the two sets of numbers inside one document. Either update the paper/thesis tables to the values in this report (recommended, because they are reproducible from the seeded scripts) or locate the original generator and dataset.

# 5. Limitations

Scores measure how well a model recovers the designed rule set from noisy synthetic inputs, not agreement with real occupants. The impurity-based importance is biased towards features with many split points and is indicative only. One random split is used; the cross-validation SD gives the spread.

# 6. Draft text for the thesis

Four regression algorithms were compared using 5-fold cross-validation on 8000 training samples. The best cross-validated model was Random Forest (10 trees, depth 5) (R2 = 0.9403). The selected Random Forest (10 trees, depth 5) achieved R2 = 0.9380, RMSE = 24.17 and MAE = 7.13 PWM units (2.8% of full scale) on the 2000-sample held-out set. PIR (38.0%) and LDR (61.7%) dominated the feature importance, consistent with occupancy-based control and daylight harvesting. [State here whether the Random Forest was chosen for accuracy, size, or both, citing Table 1 and the S1.3 footprint.]