# Step S1.6 Report: Closed-Loop Model Validation

ESP32-S3 real sensor inputs, Random Forest PWM versus rule baseline
Generated 2026-10-24 15:28

# 1. Method

The firmware reads PIR, LDR and DHT11 values every 100 ms, maps the LDR into the model's 0-1023 feature scale, runs the exported float Random Forest, and drives the LED with the model PWM. A rule baseline uses PIR=0 -> 0 and a linear 300-700 LDR ramp when PIR=1. Metrics are computed from the captured CSV files.

| Metric | Value |
| --- | --- |
| Sessions / rows | 3 / 18003 |
| Model-rule MAE (PWM) | 1.37 |
| Model-rule correlation | 0.9966 |
| PIR-off fraction | 93.30% |
| Calibration valid rows | 100.00% |
| Motion-to-model change median | 0.0 s |

Model/rule PWM and calibrated LDR over the captured sessions.

# 2. Input coverage

| Input | Minimum | Maximum |
| --- | --- | --- |
| Temperature (C) | 31.0 | 31.7 |
| Humidity (%) | 64.0 | 68.0 |
| LDR (0-1023) | 0 | 1023 |

# 3. Limitations

The rule baseline is a designed comparator, not ground truth. Results depend on the LDR calibration and the single-room test setup. Report at least three sessions and retain the raw CSV files before using these numbers in the thesis.