# Stage 2 Phase A: Development Data and Split

TinyFL federated anomaly detection; reproducible synthetic pipeline check
Generated 2026-10-24 15:37

WARNING: GENERATED FROM SIMULATED / TEST DATA (pipeline test). Do not use any number in the thesis or paper.

# 1. Data source and scope

This dataset is generated from explicit software assumptions because no Stage 2 anomaly CSV was present. It is for validating the data pipeline only. It is not measured sensor data and must not be used as thesis or paper evidence.

# 2. Synthetic label rules

| Class/type | Generated conditions |
| --- | --- |
| Normal | Temperature 24-31 C, humidity 60-80%; PIR high with probability 8% |
| temperature_event | Temperature 32-35 C; humidity 60-80% |
| humidity_event | Temperature 24-31 C; humidity 81-90% |
| pir_burst | Nominal temperature/humidity; PIR high |

DHT11-like temperature and humidity values are integer-valued. The PIR burst is a synthetic single-row proxy; it does not model a temporal motion sequence.

# 3. Dataset and split

| Type | Rows |
| --- | --- |
| normal | 4600 |
| temperature_event | 134 |
| humidity_event | 133 |
| pir_burst | 133 |

| Split | Rows | Normal | Anomaly | Anomaly rate |
| --- | --- | --- | --- | --- |
| Train | 4000 | 3680 | 320 | 8.00% |
| Test | 1000 | 920 | 80 | 8.00% |

Synthetic feature distributions by generated label.

# 4. Train-only normalization

| Feature | Train mean | Train population SD |
| --- | --- | --- |
| temperature | 27.63850 | 2.47655 |
| humidity | 70.33525 | 6.49676 |
| pir | 0.10025 | 0.30033 |

Normalization is z-score scaling; statistics were fitted on training rows only (ddof=0).

# 5. Limitations

The labels are directly constructed from the generated conditions, so model results measure recovery of these assumptions, not real-world anomaly detection. Replace this dataset with the actual Stage 2 dataset before drawing scientific conclusions or citing metrics.